PACKET TRACER LAB GUIDE

Configure IPv6 Access Control Lists (ACLs)

| TARGET DEVICES Routers R1 and R3 (Cisco IOS) | ACCESS CONTROL TYPE Named IPv6 Access Control Lists (BLOCK_HTTP & BLOCK_ICMP) |
| --- | --- |
| SECURITY THREATS MITIGATED DoS Web Refresh Attack & Distributed ICMP Flood (DDoS) | KEY SYNTAX DIFFERENCE ipv6 traffic-filter <name> in/out (replaces ip access-group) |

📌 LAB SCENARIO & OBJECTIVES: In this activity, you will mitigate two critical security events using IPv6 Access Control Lists: 1. Denial-of-Service (DoS) Attack: A malicious host on the 2001:db8:1:11::0/64 network is repeatedly refreshing web pages against Server3. You will configure an ACL on R1 named BLOCK_HTTP to block HTTP/HTTPS traffic from that network while allowing all other IPv6 traffic. 2. Distributed ICMP Ping Flood (DDoS): Multiple unknown external IPv6 addresses are flooding Server3 with ICMP echo requests. You will configure an ACL on R3 named BLOCK_ICMP placed closest to Server3 to filter all inbound ICMP packets.

IPv6 Addressing Reference Table

| Device | Interface | IPv6 Address / Prefix | Default Gateway | Connected Role / Topology Position |
| --- | --- | --- | --- | --- |
| PC1 | NIC | 2001:db8:1:10::10/64 | fe80::1 | Legitimate Workstation (R1 LAN - G0/0) |
| PC2 | NIC | 2001:db8:1:11::11/64 | fe80::1 | Compromised / Source Workstation (R1 LAN - G0/1) |
| Server3 | NIC | 2001:db8:1:30::30/64 | fe80::30 | Target Web & ICMP Server (Connected to R3 LAN) |
| R1 | G0/1 | 2001:db8:1:11::1/64 | fe80::1 | Default Gateway for PC2 Network (Target for BLOCK_HTTP) |
| R3 | G0/0 | 2001:db8:1:30::1/64 | fe80::30 | Default Gateway for Server3 Network (Target for BLOCK_ICMP) |

⚠️ KEY IPV6 ACL ARCHITECTURE & COMMAND SYNTAX: Key differences between IPv4 and IPv6 ACLs in Cisco IOS: • In IPv6, ALL access lists are named. There are no numbered IPv6 ACLs. • The global command syntax is: ipv6 access-list <name> • To apply an IPv6 ACL to an interface, use: ipv6 traffic-filter <name> in¦out (unlike IPv4's ip access-group). • IPv6 ACLs include two implicit permit statements for Neighbor Discovery (NDP): permit icmp any any nd-na and permit icmp any any nd-ns, followed by an implicit deny ipv6 any any.

Part 1: Configure, Apply, and Verify the First IPv6 ACL (BLOCK_HTTP)

Step 1: Create the Named IPv6 ACL 'BLOCK_HTTP' on Router R1

• Log in to Router R1 and enter Global Configuration mode: configure terminal

• Create the named IPv6 access list using: ipv6 access-list BLOCK_HTTP (case-sensitive).

• Enter Statement 1: Block HTTP (port 80 / www) destined for Server3: deny tcp any host 2001:db8:1:30::30 eq www

• Enter Statement 2: Block HTTPS (port 443) destined for Server3: deny tcp any host 2001:db8:1:30::30 eq 443

• Enter Statement 3: Allow all other IPv6 traffic to pass: permit ipv6 any any

• Exit configuration mode.

R1> enable R1# configure terminal R1(config)# ipv6 access-list BLOCK_HTTP R1(config-ipv6-acl)# deny tcp any host 2001:db8:1:30::30 eq www R1(config-ipv6-acl)# deny tcp any host 2001:db8:1:30::30 eq 443 R1(config-ipv6-acl)# permit ipv6 any any R1(config-ipv6-acl)# exit

Step 2: Apply BLOCK_HTTP to the Inbound Interface Closest to the Source

• Identify the source network of the DoS attack: 2001:db8:1:11::0/64 (PC2).

• The interface directly connecting the 2001:db8:1:11::0/64 network on R1 is GigabitEthernet 0/1.

• Enter interface configuration mode for G0/1.

• Apply the access list inbound using: ipv6 traffic-filter BLOCK_HTTP in

• Save the running configuration.

R1(config)# interface GigabitEthernet 0/1 R1(config-if)# ipv6 traffic-filter BLOCK_HTTP in R1(config-if)# end R1# copy running-config startup-config Destination filename [startup-config]? [Enter] Building configuration... [OK]

Step 3: Verify Part 1 ACL Implementation & Operation

• Test 1 (PC1 Legitimate Web Traffic): Open web browser on PC1 and navigate to http://2001:db8:1:30::30 or https://2001:db8:1:30::30 -> Web page MUST appear.

• Test 2 (PC2 Blocked Web Traffic): Open web browser on PC2 and navigate to http://2001:db8:1:30::30 or https://2001:db8:1:30::30 -> Web page MUST be blocked (Request timeout).

• Test 3 (PC2 Non-HTTP Traffic): From PC2 Command Prompt, ping Server3 (ping 2001:db8:1:30::30) -> Ping MUST succeed (Allowed by permit ipv6 any any).

• Review match counters on R1 using show ipv6 access-list BLOCK_HTTP.

# PC1 Web Browser Test: URL: http://2001:db8:1:30::30 --> Web page successfully loaded! (Allowed) # PC2 Web Browser Test: URL: http://2001:db8:1:30::30 --> Request Timeout / Blocked! (Denied by ACL) # PC2 Command Prompt Ping Test: PC2> ping 2001:db8:1:30::30 Pinging 2001:db8:1:30::30 with 32 bytes of data: Reply from 2001:db8:1:30::30: bytes=32 time<1ms Ping statistics for 2001:db8:1:30::30: Packets: Sent = 4, Received = 4, Lost = 0 (0% loss) # Verify ACL Hit Counters on R1: R1# show ipv6 access-list BLOCK_HTTP IPv6 access list BLOCK_HTTP deny tcp any host 2001:DB8:1:30::30 eq www (matches) deny tcp any host 2001:DB8:1:30::30 eq 443 permit ipv6 any any (matches)

Part 2: Configure, Apply, and Verify the Second IPv6 ACL (BLOCK_ICMP)

Step 1: Create the Named IPv6 ACL 'BLOCK_ICMP' on Router R3

• Log in to Router R3 and enter Global Configuration mode: configure terminal

• Create the named IPv6 access list using: ipv6 access-list BLOCK_ICMP (case-sensitive).

• Enter Statement 1: Block all ICMP traffic from any host to any destination: deny icmp any any

• Enter Statement 2: Allow all other IPv6 traffic to pass: permit ipv6 any any

• Exit configuration mode.

R3> enable R3# configure terminal R3(config)# ipv6 access-list BLOCK_ICMP R3(config-ipv6-acl)# deny icmp any any R3(config-ipv6-acl)# permit ipv6 any any R3(config-ipv6-acl)# exit

Step 2: Apply BLOCK_ICMP to the Interface Closest to the Destination

• Because the ICMP ping flood is distributed and can originate from any network, the filter must be placed closest to the protected destination (Server3).

• The interface directly attached to the Server3 subnet on R3 is GigabitEthernet 0/0.

• Enter interface configuration mode for G0/0.

• Apply the access list outbound to filter traffic heading toward Server3: ipv6 traffic-filter BLOCK_ICMP out

• Save the running configuration.

R3(config)# interface GigabitEthernet 0/0 R3(config-if)# ipv6 traffic-filter BLOCK_ICMP out R3(config-if)# end R3# copy running-config startup-config Destination filename [startup-config]? [Enter] Building configuration... [OK]

Step 3: Verify Part 2 ACL Implementation & Operation

• Test 1 (PC2 Ping Server3): Ping 2001:db8:1:30::30 from PC2 -> Ping MUST FAIL (Destination unreachable or timeout).

• Test 2 (PC1 Ping Server3): Ping 2001:db8:1:30::30 from PC1 -> Ping MUST FAIL (Filtered by R3 BLOCK_ICMP).

• Test 3 (PC1 Web Access to Server3): Open web browser on PC1 and navigate to http://2001:db8:1:30::30 -> Website MUST display successfully.

• Review match counters on R3 using show ipv6 access-list BLOCK_ICMP.

# PC2 Ping Test to Server3: PC2> ping 2001:db8:1:30::30 Pinging 2001:db8:1:30::30 with 32 bytes of data: Request timed out. Ping statistics: Packets: Sent = 4, Received = 0, Lost = 4 (100% loss) # <-- BLOCKED! # PC1 Ping Test to Server3: PC1> ping 2001:db8:1:30::30 Pinging 2001:db8:1:30::30 with 32 bytes of data: Request timed out. Ping statistics: Packets: Sent = 4, Received = 0, Lost = 4 (100% loss) # <-- BLOCKED! # PC1 Web Browser Test to Server3: URL: http://2001:db8:1:30::30 --> Web page successfully loaded! (Allowed) # Verify ACL Hit Counters on R3: R3# show ipv6 access-list BLOCK_ICMP IPv6 access list BLOCK_ICMP deny icmp any any (8 matches) permit ipv6 any any (matches)

Step 4: Check Results in Packet Tracer

• Click the Check Results button at the bottom of the Packet Tracer lab window.

• Verify that your completion score displays 100%.

• Review the Assessment Items tab to confirm all ACL statements, traffic-filter assignments, and directions are correct.

Comprehensive IPv6 ACL Policy Verification Matrix

| Source Host | Target Destination | Traffic / Protocol Tested | Expected Result | Enforcing Router & Filter |
| --- | --- | --- | --- | --- |
| PC1 (1:10::10) | Server3 (1:30::30) | HTTP (Port 80) / HTTPS (Port 443) | ✅ SUCCESS (Allowed) | Permitted (Bypasses R1 G0/1 filter) |
| PC1 (1:10::10) | Server3 (1:30::30) | ICMP Echo (Ping) | ❌ FAILS (Blocked) | R3 G0/0 Outbound (BLOCK_ICMP) |
| PC2 (1:11::11) | Server3 (1:30::30) | HTTP (Port 80) / HTTPS (Port 443) | ❌ FAILS (Blocked) | R1 G0/1 Inbound (BLOCK_HTTP) |
| PC2 (1:11::11) | Server3 (1:30::30) | ICMP Echo (Ping) [After Part 1] | ✅ SUCCESS (Allowed) | R1 BLOCK_HTTP permits other IPv6 |
| PC2 (1:11::11) | Server3 (1:30::30) | ICMP Echo (Ping) [After Part 2] | ❌ FAILS (Blocked) | R3 G0/0 Outbound (BLOCK_ICMP) |

📌 LAB COMPLETION SUMMARY: Lab completed successfully. You configured two named IPv6 access control lists, bound them using 'ipv6 traffic-filter' at optimal network boundaries (source vs. destination), and verified policy enforcement across web browsers, ICMP diagnostics, and access list hit statistics.